# 1) System at a glance

Goal: Pulse a capacitive water-cell (~1 nF, 0.0625” gap, sub-ppm DI) at 1–100 kHz with high pulse amplitude and ≤50% duty, using an Eimac 8877 (3CX1500A7) triode in grounded-grid, cathode-drive. The triode sits on a 3 kV plate deck (Spellman SL3PN1200), with a 25 mH RFC and a large HV coupling capacitor (Cc ≈ 10 nF) to the cell. The cathode is driven by a low-voltage SiC half-bridge (≈100–120 V bus) to modulate conduction cleanly and repeatedly.

Why this partitioning?

- The HV deck gives DC plate headroom and energy.

- The cathode pulser (fast, low-voltage) shapes current through the tube without shoving the whole pulse edge into a 3 kV supply.

- The large Cc >> Ccell ensures most pulse voltage appears across the cell while the plate sees controlled edges.

# 2) Electrical big-picture (blocks & nets)

Blocks

- HV Deck (3 kV): Spellman SL3PN1200 → RFC (25–50 mH total) → plate (anode).

- Triode (8877): grounded grid, heated filament (~5 V, ~10–12 A).

- Plate–Cell coupling: Cc ≈ 10 nF (two 4.8 nF / 20 kV doorknobs in parallel) from plate to cell electrode.

- Cathode pulser (LV): 120 V bus half-bridge with C3M0065090D SiC FETs, UCC21520 dual isolated driver, fast RC/TVS networks.

- Cell: ~1 nF effective capacitance; optional Rs (few hundred ohms) for damping; measurement points for V/I.

Key nets (low-voltage side)

- BUS+ / BUS- (≈100–120 V DC bus for half-bridge)

- MID_OUT (half-bridge switch node → cathode feedthrough)

- MID_RET (twisted-pair return to the cathode feedthrough) — net-tied to BUS- at one star point

- INA / INB / DISABLE (logic into UCC21520 primary)

- +5V (UCC VCCI & logic), +15V_A / COM_A, +15V_B / COM_B (isolated driver rails)

Key nets (HV deck)

- HV_PLATE, HV_RET

- GRID_RING, GRID_RET (measurement/bleed only; grid is at RF ground in GG)

- FILAMENT_5V (to heavy-copper or off-board SSR path)

# 3) What LTspice is telling us (first-order model)

We’ve been simulating a simplified grounded-grid stage with:

- VHV = 3.2 kV, RFC = 25 mH

- Cc = 10 nF, Ccell = 1 nF, optional Rdamp ≈ 220–330 Ω

- Cathode pulser modeled as a behavioral PULSE (≈90–120 V amplitude), TR/TF ≈ 2 µs, FREQ swept 1–100 kHz, DUTY 10–50%

- Measurements: Ip_avg, Ig_avg, Pp_avg, Vcell_pp, Vcell_rms over steady-state window

Key observations from the runs you captured:

- The cell is dominantly capacitive; its reactance is Xc = 1/(2π f C).

  - For Ccell = 1 nF: Xc ≈ 159 kΩ (1 kHz) → 1.59 kΩ (100 kHz).

  - So, increasing frequency boosts cell current and V_rms (to a point), while duty raises average plate dissipation.

- TR/TF = 2 µs (the param you used) is a good bring-up value: it limits Ip spikes and tames mid-node ringing while we tune hardware.

- A snubber from plate to ground (10–33 nF ∥ 1–3 kΩ) reduces plate overshoot and moves energy into a predictable resistor.

  - Larger Csn → calmer plate voltage but rising snubber watts; R ≈ 1.5–2.2 kΩ is a practical zone with 1–5 W average in the sims at 10–50% duty/10–100 kHz.

- Duty sweep (10 → 50%): plate average power scales roughly with duty; for bring-up keep to ≤20% duty until thermal steady state is measured.

Bottom line from the sims: the architecture behaves as intended; frequency is your main knob for cell “drive level,” duty is your avg-power knob, and the plate snubber is your safety valve for transients.

# 4) Hardware you’ve built toward (BOM-anchored)

Half-bridge & driver (PWR board)

- FETs: 2× Wolfspeed C3M0065090D (TO-247-3), pins: 1=G, 2=D, 3=S.

- Driver: UCC21520DW (SOIC-16W), VCCI=+5 V, OUTA→QH, OUTB→QL.

- Driver decoupling: VCCI 0.1 µF // 1 µF; VDDA/VDDB each 0.1 µF // 1 µF to their COMs.

- Input shaping: INA/INB each 100 Ω series + 100–220 pF to GND at the pin.

- Deadtime: RDT ≈ 22 kΩ (≈220 ns), optional C_DT ≤ 1 nF.

- Gate networks (each device):

  - Rg_on = 5.1 Ω series (you also have 3.3/10 Ω for tuning)

  - UF4007 diode from gate to OUTx (anode at gate, cathode at OUT) with optional Roff 0–1 Ω in series

  - P6KE18CA (bi-dir) gate-to-source clamp

  - 2.2 kΩ gate-to-source bleed

- Mid-node protection: 1.5KE170A TVSs (2–4 in parallel): cathode → MID_OUT, anode → BUS-.

- DC-link: multiple CDE 940C 1 µF/1.6 kV films in parallel BUS+↔BUS-. Keep two tight to the FET legs (commutation loop).

- MID_RET: separate return in the cable; Net-Tie-2 to BUS- at one star point near the FETs.

HV deck side

- RFC: Hammond 159ZC (60 mH/2 A) and/or 159ZE (28 mH/3 A) variants you purchased (target ≈25–50 mH effective).

- Plate–cell coupling: 2× 4.7–4.8 nF/10–20 kV doorknobs in parallel → ~9.4–9.6 nF as Cc.

- Plate snubber (to ground): your 4.7 nF/10 kV discs (CGP1C472) paralleled to ~9.4 nF, with RH-50 series resistor 1.5–3.0 kΩ/50 W.

Control / support (CTRL board)

- +5 V supply: Mean Well LRS-150F-5 (you bought).

- Isolated +15 V rails: 2× Recom R12P215S (you bought) to feed UCC secondaries.

- Interlocks & enables: LM311N comparators and Baomain SSR-10DD for Spellman enable and for precharge bypass after ~1 s (LM311 RC timer using 10 MΩ VR68 + 0.1–1.0 µF WIMA).

- Filament/grid monitors: 2× LEM LA55-P with 100 Ω / 5 W burden and OPA277P buffer → controller ADC.

- Fan control: open-collector PWM using LM311 if needed (no new parts).

- TVS & diodes: 1.5KE170A (mid-node), P6KE18CA (gate), UF4007 (gate branches, flybacks).

# 5) KiCad status (what’s placed, what’s wired, what’s left)

You have on PWR:

- UCC21520 with VCCI decouplers and DT network (RDT ≈ 22 kΩ; optional C_DT added footprint)

- INA/INB RC deglitch right at pins (100 Ω + 100–220 pF to GND)

- Two SiC FETs with:

  - Correct pin mapping (G-D-S = 1-2-3)

  - Rg_on 5.1 Ω in series OUT→G

  - UF4007 + Roff branch (anode at gate, cathode at OUT)

  - P6KE18CA and 2.2 kΩ gate-to-source

- 1.5KE170A TVSs MID_OUT→BUS- (band to MID_OUT)

- 940C films BUS+↔BUS- (five total, two very close to the FET legs)

- MID_RET as its own net + Net-Tie-2 to BUS- (single-point)

To finish on PWR:

- Drop test points: TP_BUS+, TP_BUS-, TP_MID, TP_QH_G, TP_QL_G, TP_VCCI, TP_VDDA, TP_VDDB.

- Place RC snubber footprint at MID_OUT→BUS- (2.2 Ω/5 W + 0.1–0.33 µF/1.6 kV) in case ringing persists after bring-up.

- Confirm Kelvin returns: COM_A to QH source (MID_OUT) and COM_B to QL source (BUS-) with dedicated short traces (not the power plane).

CTRL board plan (parts you already own):

- Power header (+5 V, GND), +15V_A/COM_A, +15V_B/COM_B output headers to PWR

- LM311 blocks: Interlock → OK_L; Filament warm timer → /FIL_WARM; AND via diode-OR to drive SSR-HV input

- LM311 + RC timer to drive SSR-PRE after ~1 s to short the 50 Ω precharge

- LA55-P + OPA277P + 100 Ω burden → FIL_I_MON and GRID_I_MON to controller

- Fan header (optional LM311 open-collector PWM sink)

# 6) Bring-up plan (step-by-step, safe & repeatable)

Stage 0 — Offline checks

- Continuity: no shorts BUS+↔BUS-, gate nets not shorted, TVS polarities right.

- Gate driver: with only +5 V applied, DISABLE low, scope OUTA/OUTB (no +15 V yet).

- Apply +15V_A/B; verify clean 0/15 V square waves at OUTA/OUTB (no FETs populated yet).

Stage 1 — Populate FETs, no HV

- Populate one FET at a time (start low-side).

- Power BUS ≈ 24–48 V (not 120 V yet). Feed INA/INB at 10–20 kHz, 10–20% duty, DT ≈ 220 ns.

- Scope MID_OUT, gate Vgs, and drain Vds (with a proper HV probe).

- Tune Roff (0 → 0.82 → 1.5 Ω) and Rg_on (3.3 → 5.1 → 10 Ω) to minimize overshoot and ringing.

- Add/use RC snubber if needed.

Stage 2 — Raise BUS to 100–120 V

- Use your precharge path: 50 Ω / 50 W, then SSR bypass after ~1 s.

- Re-check thermal—your 940C films should be cool; FETs warm but not hot.

- Confirm MID_RET star-tie is quiet (probe for ground bounce).

Stage 3 — HV deck integration

- Connect RFC → plate, Cc → cell, maintain clearances & creepage.

- Start HV at low kV, low duty/freq; confirm Ip_avg, Pp_avg in safe limits; verify plate snubber temperature.

- Walk freq up while watching grid current (Ig) and plate dissipation; keep duty ≤ 20% until temps stabilize.

- Log Vcell_pp, Vcell_rms, Ip_avg, Ig_avg, Pp_avg at each setpoint (use the same .measure windows you used in LTspice to keep apples-to-apples).

# 7) Tuning cheatsheet (what to touch when)

| Symptom | Try this (in order) |
| --- | --- |
| MID_OUT overshoot/ringing | Increase Roff (0→0.82→1.5 Ω); increase Rg_on (3.3→5.1→10 Ω); add/tune RC snubber MID→BUS-; add more 940C close-in |
| Gate ringing or false turn-on | Verify P6KE18CA G↔S; shorten COM_A/COM_B Kelvin; increase deadtime (RDT 22k→27k/33k); add ferrite bead in series with gate |
| Plate overshoot | Increase Csnub plate→ground (9.4 nF→~14 nF); lower Rsnub (3 k→2.2 k); keep wiring short to plate node |
| Average plate power too high | Reduce duty; lower freq; reduce VDRIVE; increase snubber C/R with awareness of added dissipation |
| Gate driver hot/noisy | Check VCCI decoupling & routing; confirm INA/INB have 100 Ω + 100–220 pF at pins |

# 8) Safety & construction notes

- Treat MID_OUT and plate as RF/HV nodes—short, wide copper; no daisy-chains.

- Use creepage/clearance appropriate for 3 kV (≥10–12 mm on FR-4 in humid service; more is better).

- One-point bond only: MID_RET → BUS- via Net-Tie-2. Keep COM_A/COM_B Kelvin—don’t let them share load current.

- Filament supply currents are large; keep them off the CTRL PCB copper—route through external SSR/heavy copper and only monitor via LA55-P.

- Interlocks should fail-safe (open loop = HV inhibit). Your SSR-10DD input drive is limited and protected via LM311 open-collector outputs and UF4007s.

# 9) What’s already bought (core items we depend on)

- FETs: 5× C3M0065090D

- Driver: 5× UCC21520DW

- Isolated DC/DC: 2× R12P215S (+15 V/2 W)

- Decoupling: WIMA 0.1 µF & 1 µF films; CDE 940C 1 µF/1.6 kV films

- Protection: 1.5KE170A (mid-node), P6KE18CA (gate), UF4007 (gate branch)

- Power resistors: RH-50 50 Ω (precharge), RH-50 1.5 k/3 k (snubbers/bleeds), MP930-2 k (pull-ups/loads), 100 Ω/5 W (burdens)

- Sensors & amps: 2× LEM LA55-P, 3× OPA277P

- Supplies: LRS-150F-5 (5 V), Genesys 300-5 (bus), Spellman SL3PN1200 (plate HV)

- Timing/relays: H3Y-2 timer (12 V) if desired; Baomain SSR-10DD (DC-DC SSR)

# 10) Open items / to-do

- Finish CTRL schematic (using only purchased parts):

  - LM311 blocks (Interlock, Filament warm, AND gating), SSR-HV drive, SSR-PRE timer

  - LA55-P burdens + OPA277 buffers (FIL_I_MON, GRID_I_MON)

  - Jacks: J_TO_PWR (INA/INB/DISABLE/GND), J_+15V_A/B, J_INTERLOCK, J_SPELLMAN_EN, J_FAN4, test points

- PWR layout: route with tight commutation loop; two 940C closest; snubber footprints; Kelvin returns.

- Define bring-up checklist in the repo (copy section 6) and an LTspice sweep script (your .step FREQ & .step DUTY plus plate-snubber sweep you already have).

- Bench fixtures: safe HV cabling, scope HV probes, bleed paths, shields, and labels.

## Appendix: quick math checkpoints

- Cell reactance Xc = 1/(2π f Ccell) for 1 nF:

  - 1 kHz → 159 kΩ; 10 kHz → 15.9 kΩ; 32 kHz → 4.97 kΩ; 100 kHz → 1.59 kΩ.

Expect increasing cell current with frequency (until limited by tube and coupling).

- Deadtime: DT(ns) ≈ 10 × RDT(kΩ) → 22 kΩ ≈ 220 ns (safe for bring-up).

- Precharge: at 300 V with 50 Ω, initial ≈ 6 A decays with RC set by total bus capacitance; bypass with SSR after ~1 s.

| Source | Index | Vendor SKU (if any) | MPN | Manufacturer | Description | Qty | Unit_Price_USD | Ext_Price_USD | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DigiKey | 1 | 1697-C3M0065090D-ND | C3M0065090D | Wolfspeed | SICFET N-CH 900V 36A TO-247-3 | 5 | 20.93 | 104.65 |  |  |
| DigiKey | 2 | 296-48796-1-ND | UCC21520ADWR | Texas Instruments | Isolated 2ch gate driver SOIC-16W | 5 | 4.62 | 23.1 |  |  |
| DigiKey | 3 | 945-2063-5-ND | R12P215S | Recom Power | DC/DC converter 12V→±15V 2W SIP-7 | 2 | 14.91 | 29.82 |  |  |
| DigiKey | 4 | 338-1170-ND | 940C16W1K-F | Cornell Dubilier | Film cap 1.0uF 1.6kV axial (CDE 940C) | 2 | 20.27 | 40.54 |  |  |
| DigiKey | 5 | 398-1010-ND | LA 55-P | LEM | Current transducer 50A | 2 | 32.86 | 65.72 |  |  |
| DigiKey | 6 | 541-RH05050R00FE02-ND | RH05050R00FE02 | Vishay Dale | Chassis resistor 50Ω 50W 1% | 2 | 40.51 | 81.02 | Precharge |  |
| DigiKey | 7 | 1.5KE120ACT-ND | 1.5KE120A | – | TVS diode 103VWM DO-201 | 20 | 0.605 | 12.1 |  |  |
| DigiKey | 8 | LM311NNS/NOPB-ND | LM311N/NOPB | Texas Instruments | Comparator DIP-8 (open collector) | 10 | 0.663 | 6.63 |  |  |
| DigiKey | 9 | 1866-3329-ND | LRS-150F-5 | Mean Well | AC/DC 5V 110W supply | 1 | 23.2 | 23.2 |  |  |
| DigiKey | 10 | 751-1262-5-ND | 6N137 | Vishay | Logic optocoupler 10MBd DIP-8 | 10 | 1.72 | 17.2 |  |  |
| DigiKey | 11 | 13-CFR-25JR-52-5R1CT-ND | CFR-25JR-52-5R1 | Yageo | Resistor 5.1Ω 1/4W axial | 10 | 0.034 | 0.34 | Gate R start |  |
| DigiKey | 12 | 13-CFR-25JR-52-3R3CT-ND | CFR-25JR-52-3R3 | Yageo | Resistor 3.3Ω 1/4W axial | 50 | 0.0246 | 1.23 |  |  |
| DigiKey | 13 | 13-CFR-25JR-52-10RCT-ND | CFR-25JR-52-10R | Yageo | Resistor 10Ω 1/4W axial | 50 | 0.0246 | 1.23 |  |  |
| DigiKey | 14 | UF4007CT-ND | UF4007 | onsemi | Fast diode 1000V 1A DO-41 | 20 | 0.218 | 4.36 | Gate turn-off path |  |
| DigiKey | 15 | 1928-FKP2D001001D00JSSD-ND | FKP2D001001D00JSSD | WIMA | Polypropylene film cap 100pF 100V | 10 | 0.329 | 3.29 |  |  |
| DigiKey | 16 | 1928-1656-ND | MKS2D031001A00MSSD | WIMA | Film cap 0.1uF 100V radial | 20 | 0.312 | 6.24 | Decoupling |  |
| DigiKey | 17 | 1928-MKS4D041003F00JSSD-ND | MKS4D041003F00JSSD | WIMA | Film cap 1uF 100V radial | 10 | 0.793 | 7.93 | Decoupling |  |
| DigiKey | 18 | 1.5KE170ACT-ND | 1.5KE170A | – | TVS diode 145VWM 234VC DO-201 | 20 | 0.591 | 11.82 | Mid-node clamp |  |
| DigiKey | 19 | OPA277P-ND | OPA277P | Texas Instruments | Op amp precision DIP-8 | 3 | 8.93 | 26.79 | Sense buffer |  |
| DigiKey | 20 | 541-RH0501K500FE02-ND | RH0501K500FE02 | Vishay Dale | Chassis resistor 1.5kΩ 50W 1% (E02) | 1 | 22.63 | 22.63 | Snubber/bleeder |  |
| DigiKey | 21 | 541-RH0503K000FE02-ND | RH0503K000FE02 | Vishay Dale | Chassis resistor 3kΩ 50W 1% | 1 | 31.7 | 31.7 | Snubber/bleeder |  |
| DigiKey | 22 | 541-RH0501K500FC02-ND | RH0501K500FC02 | Vishay Dale | Chassis resistor 1.5kΩ 50W 1% (C02) | 1 | 34.49 | 34.49 | Snubber/bleeder |  |
| DigiKey | 23 | 4878-P6KE18CACT-ND | P6KE18CA | – | TVS diode 18V bidir DO-204AC | 50 | 0.242 | 12.1 | Gate clamp |  |
| DigiKey | 24 | HM1554-ND | 159ZC | Hammond | Chassis inductor 60mH 2A 0.7Ω | 1 | 41.7 | 41.7 | HV choke option |  |
| DigiKey | 25 | HM4112-ND | 159ZE | Hammond | Chassis inductor 28mH 3A 0.43Ω | 1 | 41.48 | 41.48 | HV choke option |  |
| DigiKey | 26 | MP930-2.00KF-ND | MP930-2.00K-1% | Caddock | Power resistor 2kΩ 30W TO-220 | 5 | 5.23 | 26.15 | Pull-ups/load |  |
| DigiKey | 27 | WNE100FETCT-ND | WNE100FET | Vishay (Welwyn) | Resistor 100Ω 5W axial | 10 | 1.505 | 15.05 | Burdens/RC |  |
| Other | 28 | TDK Lambda Genesys GEN 300-5 | GEN300-5 | TDK-Lambda | Programmable DC supply 300V 5A 1500W | 1 | 1125.06 | 1125.06 | Bus supply |  |
| Other | 29 | SL3PN1200 | SL Series | Spellman | HV supply 0–3kV programmable | 1 |  |  | You noted you have this; price not provided |  |
| Other | 30 | – | Ceramic tube socket (8877/3CX1500A7) | – | 8877/3CX1500A7 socket | 1 |  |  | For Eimac 8877 |  |
| Other | 31 | – | 4800pF 20kV doorknob | – | HV capacitor 4.8nF 20kV | 2 | 75 | 150 | Plate–cell coupling (pair ≈9.6nF) |  |
| Other | 32 | – | 1000pF 10kV doorknob | – | HV capacitor 1.0nF 10kV | 3 |  |  | Snubbers/experiments |  |
| Other | 33 | – | H3Y-2 12V timer | Omron | Delay timer 0–60 s + base | 1 |  |  | Precharge delay option |  |
| Other | 34 | – | SSR-10DD | Baomain | DC–DC solid-state relay 10A | 1 |  |  | Precharge/HV enable |  |
| Other | 35 | – | TO-247 heatsinks set | – | Small clip-on heatsinks + hardware | 1 set |  |  | For SiC parts |  |
| Other | 36 | – | TO-247 alumina pads | – | Ceramic insulators 17×22×1 mm | 100 |  |  | Isolation for TO-247 |  |
| Other | 37 | – | Snap-in capacitor 680uF 200V | – | Electrolytic 25×30 mm | 2 |  |  | Bulk bus (pair) |  |
| Other | 38 | – | Nichicon 680uF 200V CE series | Nichicon | Electrolytic 25×40 mm | 2 |  |  | Bulk bus (pair) |  |
| Other | 39 | – | BOJACK HV ceramic kit | – | HV ceramic caps 3kV 100pF–10nF assortment | 1 |  |  | Prototyping |  |
| Other | 40 | – | Resistor kits (10W | 50W) | – | Various power resistors assortments | – |  |  | Tuning/loads |
| Other | 41 | – | 6N137 (extra) | – | Opto (duplicates of DigiKey line) | – |  |  | If bought via Amazon too |  |
| Other | 42 | – | Heatsink insulator/mount kits | – | TO-220/TO-247 hardware packs | 2 |  |  | Assembly |  |