TOBE ENERGY CORP

Techno-Economic Analysis:
Membrane-Free Hydrogen Electrolysis

Production Economics, Competitive Position, and Investment Thesis

March 2026

Version 3.0 | Currency Year: 2026 USD

CONFIDENTIAL — INVESTOR USE ONLY

TABLE OF CONTENTS

1.  Executive Summary

2.  System Boundary Definition

3.  Technical Model

3.1  Measured Performance Data

3.2  Heat and Material Balance

3.2.1  Efficiency Boundary Clarification

3.3  Three Modeling Scenarios

4.  Cost Model (CAPEX)

4.1  CAPEX Taxonomy

4.2  Installed Cost Estimates

5.  OPEX Model

5.1  Electricity

5.2  Fixed O&M

5.3  Water

5.4  Stack Replacement and Degradation

6.  Revenue and Margin Model

6.1  Contracted and LOI Pricing

6.2  Gross Margin Analysis

6.3  The On-Site Advantage

6.4  Delivered Cost by End-Use Application

6.5  Revenue Streams

7.  Financial Model / DCF

7.1  LCOH Methodology

7.2  Base Case LCOH Calculation

7.3  Three-Scenario Summary

7.4  WACC Sensitivity

7.5  45V Production Tax Credit Analysis

7.6  Zeeco Phase 1 Deployment Economics

7.7  DCF / Revenue Projections

8.  Sensitivity Analysis

8.1  Tornado Chart

8.2  Multi-Variable Scenario Analysis

8.3  Capacity Factor Sensitivity

8.4  Selling Price Sensitivity

9.  Competitive Comparison

10.  Conclusions

A.  Source File Reference

B.  Glossary

# 1. Executive Summary

Tobe Energy Corp has developed a membrane-free, catalyst-free hydrogen electrolyzer that achieves 70–72% LHV wall-to-plug efficiency across all tested conditions — competitive with or exceeding PEM electrolysis at a fraction of the capital cost. Measured performance data from 22,103 data points across four test configurations establishes a system-level specific energy consumption of 46.08 kWh/kg at the electrolyzer outlet, with a demonstrated stack-level SEC of 42.2 kWh/kg using optimized power electronics.

## Headline LCOH Results (at Electrolyzer Outlet)

| Scenario | SEC (kWh/kg) | Installed CAPEX ($/kW) | Elec ($/kWh) | LCOH ($/kg) | Basis |
| --- | --- | --- | --- | --- | --- |
| Downside | 47.5 | 1,200 | 0.045 | $3.61 | Measured bench performance, higher electricity/CAPEX, 75% CF |
| Base Case | 46.5 | 1,000 | 0.040 | $2.79 | System-level SEC, large-power industrial tariff |
| Upside | 42.0 | 800 | 0.035 | $2.06 | Stack SEC with optimized LLC topology, PSO renewable rate |

All three scenarios fall below the DOE/NREL PEM electrolysis benchmark of $4–7/kg (unsubsidized).

45V Production Tax Credit impact (scenario-dependent):

| 45V Scenario | PTC Value | Compliance Cost | Effective Base LCOH |
| --- | --- | --- | --- |
| Full Tier 1 ($3.00/kg) | -$3.00 | +$0.10 | -$0.11/kg |
| Partial Tier 2 ($1.00/kg) | -$1.00 | +$0.05 | $1.84/kg |
| No PTC (baseline) | $0 | $0 | $2.79/kg |

## The On-Site Advantage

The comparison that matters to customers is delivered cost, not production cost. DOE data shows 75–85% of delivered hydrogen cost is transport and storage:

| Hydrogen Source | Production Cost | Delivered Cost | Tobe On-Site |
| --- | --- | --- | --- |
| Grey hydrogen (SMR) | $1–2/kg | $15–25/kg | — |
| Cylinder delivery (Airgas quote, Sept 2024) | — | $121.31/kg | — |
| PEM electrolysis (renewable) | $4–7/kg | $8–15/kg | — |
| Tobe Energy (base case) | $2.79/kg | $0/kg delivery | $2.79/kg total |

Tobe produces on-site, eliminates delivery entirely for low-pressure industrial customers, and sells at $15–30/kg with 81–91% gross margins validated by real contracts and LOIs across seven industry sectors.

## Key Differentiators

Wall-to-plug efficiency: All reported efficiencies are true AC-in → H₂-out measurements. Most competitors report only stack efficiency (DC → H₂), which is 10–15% higher. Tobe's 70–72% LHV wall-to-plug exceeds typical PEM wall-to-plug of 57–67% LHV.

No membrane: Eliminates Nafion ($500–800/m²) and associated degradation-driven replacement cycles.

No rare earth catalysts: Uses stainless steel construction. No iridium/platinum ($1,000–3,000/kW for PEM precious metals eliminated).

No cooling system: Operates at 27–28°C near ambient. Eliminates heat exchangers, pumps, and coolant loops.

TDS tolerance: Operates across 25–10,000+ ppm TDS, enabling standard RO instead of expensive deionization.

# 2. System Boundary Definition

## Reporting Boundary

Per Agora Energiewende best practice and the methodology recommended in the TEA Deep Research Report, LCOH is reported at the electrolyzer outlet for comparability with published benchmarks. Delivered-cost adders by end-use application are shown separately in Section 6.4.

## System Includes

| Component | Description | Source |
| --- | --- | --- |
| Electrolyzer stack | Membrane-free, stainless steel construction, 304 SS | TOBE-FEED-PKG-001 |
| Balance of plant | Power electronics (LLC resonant topology), controls | TOBE FEED Package |
| Water treatment | Integrated RO/DI water treatment | January 2026 quote |
| Gas separation | H₂ purification and dual-bed dryer train to ≥99.99% | January 2026 quote; TOBE-PFD-001 |
| Instrumentation & safety | Complete instrumentation and safety interlocks | January 2026 quote |
| Electrical interconnection | 480V 3Φ 60Hz, transformer, MCC | TOBE FEED Package |
| Enclosure HVAC | ~10 kW total (minimal due to near-ambient operation) | TOBE FEED Package |

## System Excludes

Compression beyond outlet pressure (2.75 bar(g) / ~40 psig at stack)

Bulk hydrogen storage

Transport and distribution

End-use dispensing equipment

Compression and storage are separate scope items, confirmed by the January 2026 equipment quote which lists these as TBD line items distinct from the electrolyzer system.

## Reference Design

The Zeeco Advanced Research Complex (ARC) Phase 1 deployment serves as the reference design:

Location: Broken Arrow, Oklahoma

Configuration: 12× T-25 electrolyzer units

Total capacity: 300 kg H₂/day at 600 kW connected load

H₂ disposition: Direct to Zeeco flare/burners (no storage in Phase 1)

Status: Contracted, deployment Q4 2026

Source: TOBE FEED Package – Zeeco ARC; TOBE-PFD-001

# 3. Technical Model

## 3.1 Measured Performance Data

All efficiency data derives from laboratory testing of the 6 kW and 15 kW prototype electrolyzers, documented in federal grant submission (Navy SBIR N242-070) and internal test records.

Source: ENG_EfficiencyCalculations.xlsx; ENG_PreCommit_SampleOperatingData.xlsx

### Four Test Configurations

| Test Run | Avg Power (W) | Duration (hrs) | Avg Flow (LPM) | Total H₂ Mass (kg) | SEC (kWh/kg) | LHV Efficiency | HHV Efficiency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6 kW Low TDS | 6,012 | 6 | 9.8 | 0.783 | 46.08 | 72.37% | 85.5% |
| 6 kW High TDS | 6,143 | 6 | 9.7 | 0.775 | — | 70.11% | 82.9% |
| 15 kW Low TDS | 14,972 | 6 | 24.3 | 1.942 | — | 72.06% | 85.1% |
| 15 kW High TDS* | 15,096 | 2 | 23.5 | 0.939 | 48.2* | 69.15% | 81.7% |

\ 15 kW High TDS test ended early (2 hours) due to diode failure. Daily rate extrapolation not meaningful. SEC derived from measured LHV efficiency (33.33 ÷ 0.6915). Efficiency calculated over actual test duration.*

Source: ENG_EfficiencyCalculations.xlsx; ENG_GRANT_NavySBIR_N242-070.pdf

Critical context:

These are true wall-to-plug efficiencies (AC mains → H₂ gas out). The test setup measures AC power input at the wall outlet and H₂ mass output, capturing all conversion losses including the DC bench power supply. Most electrolyzer manufacturers report stack-only efficiency (DC → H₂), which omits power electronics losses of 10–15%. Tobe's reported numbers are directly comparable to the DOE/NREL "system" efficiency metric.

Tests used a standard DC bench power supply, not an optimized power topology. The bench supply introduces additional rectification losses. Tobe's production units use a resonant LLC power topology that has demonstrated 94% HHV efficiency in controlled testing. The measured bench results represent a conservative floor, not optimistic projections.

### Operating Data Summary (6 kW Low TDS Run)

The primary test dataset contains 22,103 data points at 5-second intervals from the 6 kW Low TDS run (August 18, 2024).

Source: ENG_PreCommit_SampleOperatingData.xlsx

| Parameter | Value | Unit | Notes |
| --- | --- | --- | --- |
| Run ID | 6kW_Low_TDS | — | Best-performing bench test |
| Date | August 18, 2024 | — | — |
| Total H₂ produced | 782.66 | g | Over test duration |
| AC power (steady state) | ~6,000 | W | Stable ±60 W range |
| Stack temperature | 27–28 | °C | Near ambient, no thermal runaway |
| Gas outlet temperature | 27.1 | °C | Consistent with stack temp |
| Ambient temperature | 24.1 | °C | Laboratory environment |
| Operating pressure | 25.3 | psig | 2.75 bar(g) at stack |
| H₂ flow rate | 9.9 | LPM | Stabilized after initial ramp |
| Duty cycle | 60% | — | Pulsed operation |
| Pulse frequency | 16,400 | Hz | — |
| Measured SEC | 46.08 | kWh/kg | Wall-to-plug (system-level) |

The data shows remarkable operational stability: stack temperature holds within ±1°C of ambient across the entire test run, with no evidence of transient oscillations, thermal drift, or performance degradation over 30+ hours of logged data.

## 3.2 Heat and Material Balance

Source: ENG_HMB_HeatAndMaterialBalance.xlsx; NOTES_heat_material_balance.md

### Single T-25 Unit (25 kg/day Design Basis)

| Stream | Flow Rate | Temperature | Pressure | Composition |
| --- | --- | --- | --- | --- |
| City water in | 15.6 kg/hr | 24°C | 2 bar(g) | Municipal supply |
| RO permeate | 9.4 kg/hr | 28°C | 2 bar(g) | Purified water (80% RO recovery) |
| RO brine (waste) | 6.25 kg/hr | 24°C | 2 bar(g) | Reject stream |
| Stack gas outlet | 9.31 kg/hr | 27°C | 2.75 bar(g) | H₂ + O₂ mixed gas |
| Pure H₂ product | 1.042 kg/hr | 27°C | 3.08 bar(g) | ≥99.99% H₂ |
| Pure O₂ byproduct | 8.94 kg/hr | 27°C | 3.08 bar(g) | Separated O₂ |

Energy inputs:

Electrolyzer stack: 1,154 kWh/day (48.1 kW continuous)

Booster pump: 2 kWh/day (negligible)

Total: 1,156 kWh/day → 46.2 kWh/kg H₂

Water consumption: 15.6 kg city water → 9.4 kg RO permeate per 1.042 kg/hr H₂ = 9.0 kg water per kg H₂ (vs. PEM requiring 9–10 kg ultra-pure DI water per kg H₂)

### 3.2.1 Efficiency Boundary Clarification

The T-25 system has two distinct efficiency levels that must be clearly distinguished:

| Metric | Stack SEC | System SEC | Difference |
| --- | --- | --- | --- |
| kWh/kg H₂ | 42.2 | 46.0–46.5 | ~4 kWh/kg |
| HHV efficiency | 93.4% | 85–86% | ~8 pp |
| Boundary | Electrolyzer cell only | AC service entrance → H₂ at outlet | BOP parasitic load |

Stack SEC (42.2 kWh/kg): Energy consumed by the electrolyzer cells alone, validated by the Zeeco PFD design basis (TOBE-PFD-001) and the January 2026 equipment quote. This is the electrolyzer-only efficiency — the metric most PEM manufacturers report.

System SEC (46.0–46.5 kWh/kg): Total AC energy from service entrance to hydrogen at the outlet flange, including all BOP parasitic loads: RO/DI water treatment pumps, gas separation and purification, dual-bed dryer train, industrial PLC/HMI controls, safety interlock systems, and enclosure HVAC. This is the true wall-to-plug metric used throughout this TEA.

The ~4 kWh/kg difference between stack and system SEC represents the BOP parasitic load. This is confirmed by two independent measurements:

Bench testing: Measured 46.08 kWh/kg at the AC wall outlet with all conversion losses captured (ENG_PreCommit_SampleOperatingData.xlsx)

Heat & material balance: 1,154 kWh/day ÷ 25 kg/day = 46.2 kWh/kg (ENG_HMB)

The base case SEC of 46.5 kWh/kg used in this TEA is a system-level number, deliberately set slightly above the best measured system result (46.08 kWh/kg) to provide conservatism. It should be compared to other TEAs' system-level SEC values (e.g., DOE PEM benchmark of ~57.5 kWh/kg system-level), not to stack-only metrics.

### Multi-Unit Deployment (Zeeco Phase 1: 12× T-25)

The H&MB file contains a 15-unit sheet; however, Zeeco Phase 1 was revised to 12 units per the FEED package. At 12 units:

| Parameter | Value | Source |
| --- | --- | --- |
| Total H₂ output | 300 kg/day (12.5 kg/hr) | Scaled from single-unit H&MB |
| Total stack energy | 13,848 kWh/day | 12 × 1,154 kWh/day |
| Electrolyzer demand | 576 kW (12 × 48 kW) | TOBE FEED Package |
| Auxiliary power | ~50 kW (RO, HVAC, controls) | TOBE FEED Package |
| Total demand | ~626 kW | TOBE FEED Package |
| Transformer | 750 kVA (85% loaded) | TOBE FEED Package |
| SEC (system-level) | 46.2 kWh/kg | Consistent with single unit |

## 3.3 Three Modeling Scenarios

| Scenario | Efficiency (HHV) | SEC (kWh/kg) | Basis | Source |
| --- | --- | --- | --- | --- |
| Downside | ~83% | 47.5 | Measured bench power supply performance at high TDS | ENG_EfficiencyCalculations (6 kW High TDS: 82.9% HHV) |
| Base Case | 85% | 46.5 | System-level SEC with conservative margin above measured | ENG_PreCommit (46.08 kWh/kg measured), derated slightly |
| Upside | 94% | 42.0 | Stack-level SEC with optimized LLC topology, clean water | TOBE-PFD-001 (42.2 kWh/kg deployment basis) |

The base case SEC of 46.5 kWh/kg is a system-level number (see Section 3.2.1), deliberately set slightly above the best measured system result (46.08 kWh/kg). The upside of 42.0 kWh/kg represents the stack-level SEC validated by the Zeeco PFD design basis.

# 4. Cost Model (CAPEX)

## 4.1 CAPEX Taxonomy

Following the DOE/NREL taxonomy recommended in the TEA Deep Research Report, CAPEX is disaggregated into direct equipment and indirect/soft costs.

### Direct Equipment

| Component | Tobe | PEM Equivalent | Tobe Advantage |
| --- | --- | --- | --- |
| Electrolyzer stack | Stainless steel (304 SS), no membrane, no catalysts | Nafion membrane ($500–800/m²) + Ir/Pt catalysts ($1,000–3,000/kW) | Eliminates ~50% of PEM stack cost |
| Power electronics | LLC resonant topology, 480V input | Comparable rectifier/power stage | Similar cost |
| Water treatment | Standard RO system (tolerates high TDS) | Deionized/ultra-pure water system | Lower cost; simpler maintenance |
| Gas separation | H₂/O₂ separation/purification | Membrane provides inherent separation | Additional cost, offset by membrane elimination |
| Thermal management | None required — operates at 27–28°C | Active cooling: heat exchangers, pumps, coolant | Eliminated entirely |

Figure 1: Tobe vs. PEM Component Cost Comparison

Source: TOBE-FEED-PKG-001 (materials specification); TOBE FEED Package (equipment scope)

### What Tobe Eliminates vs. PEM

| PEM Component | Typical PEM Cost Contribution | Tobe Status |
| --- | --- | --- |
| Proton exchange membrane (Nafion) | $500–800/m²; major stack cost driver | Eliminated — membrane-free design |
| Iridium/platinum catalysts | $1,000–3,000/kW for precious metals | Eliminated — stainless steel electrodes |
| Active cooling system | Moderate (heat exchangers, circulation pumps, coolant) | Eliminated — near-ambient operation (27°C) |
| Deionized water system | Moderate (DI/UPW treatment train) | Replaced with standard RO — tolerates 25–10,000+ ppm TDS |

## 4.2 Installed Cost Estimates

### Equipment Pricing (January 2026)

The T-25 equipment price of $25,000 ($500/kW) is an all-in production unit that already includes:

Complete electrolyzer with integrated power electronics

Industrial PLC/HMI controls

Integrated RO/DI water treatment system

H₂ purification and dual-bed dryer train

Complete instrumentation and safety interlocks

480VAC 3-phase electrical interface

| Product | Rated Power | H₂ Output | Equipment Price | $/kW (equipment) |
| --- | --- | --- | --- | --- |
| T-25 | 50 kW | 25 kg/day | $25,000 | $500/kW |
| T-125 | 250 kW | 125 kg/day | $120,000 | $480/kW |

The T-125 demonstrates cost reduction at scale ($480/kW vs. $500/kW), consistent with the financial model's 12% annual manufacturing learning rate.

Equipment pricing reflects early-market breakeven positioning (no margin on equipment for anchor deployments). Production-volume pricing will incorporate margin while remaining below PEM equipment costs due to structural material advantages (no membrane, no precious metals, stainless steel construction).

### Installed CAPEX Breakdown

The gap between $500/kW equipment cost and $1,000/kW installed cost is decomposed into auditable categories:

| Cost Category | $/kW | % of Installed | Notes |
| --- | --- | --- | --- |
| Electrolyzer unit (all-in equipment) | $500 | 50% | Includes stack, power electronics, PLC/HMI, RO/DI, purification, dryer, instruments, interlocks, 480V interface |
| Plant control / SCADA | $60 | 6% | $30,000 per ~500 kW cluster |
| Electrical interconnection | $100 | 10% | Transformer, MCC, breaker gear, conduit, conductor runs |
| EPC / installation / commissioning | $100 | 10% | Labor, piping, startup |
| Remote monitoring system | $32 | 3% | $16,000 per ~500 kW cluster |
| Extended warranty & consumables | $36 | 4% | First-year spares/service package |
| Site preparation & contingency | $100 | 10% | Pad, enclosure, permits, owner's costs |
| Unallocated margin | $72 | 7% | Conservative reserve |
| Total installed | $1,000 | 100% |  |

Context vs. PEM benchmarks (DOE/NREL):

PEM equipment only (manufactured + markup): $800–1,350/kW (NREL 2024 bottom-up analysis)

PEM installed cost: $1,400–2,500/kW (DOE 2024 program record)

Tobe equipment cost ($500/kW) is 37–63% below PEM equipment-only cost

Tobe installed cost ($1,000/kW) is below PEM uninstalled (equipment) cost

This structural CAPEX advantage stems from the elimination of the three most expensive PEM components (membrane, precious metal catalysts, cooling system) and the use of commodity stainless steel construction.

### Indirect / Soft Costs

| Cost Category | Notes | Source |
| --- | --- | --- |
| EPC / installation | Zeeco ARC project provides real installation scope | TOBE FEED Package |
| Commissioning | Included in FEED schedule | TOBE FEED Package |
| Permitting | ISO 22734 certification planned Q2–Q3 2026 | TOBE-FEED-PKG-001 |
| Electrical interconnection | 750 kVA transformer, MCC, conduit | TOBE FEED Package |
| Building/enclosure | 3,000–4,000 sq ft (Zeeco provides) | TOBE FEED Package |

### Manufacturing Learning Curve

The financial model projects a 12% annual cost reduction from manufacturing learning effects, consistent with electrolyzer industry learning rates documented by IRENA (2020).

# 5. OPEX Model

## 5.1 Electricity — The Dominant Cost Driver

Electricity represents ~67% of total LCOH at base case, consistent with the 55–70% range documented across nine TEA exemplars in the deep research report.

| Parameter | Value | Source |
| --- | --- | --- |
| Electricity price (large-power industrial tariff) | $0.04/kWh | PSO large-power industrial tariff |
| Annual consumption per T-25 (85% CF) | 360,653 kWh | 7,756 kg × 46.5 kWh/kg |
| Annual electricity cost per T-25 | $14,426 | — |
| Electricity cost per kg H₂ | $1.86/kg | 67% of base case LCOH |

Electricity price basis: Tobe facilities qualify for the PSO (Public Service Company of Oklahoma) large-power industrial tariff at $0.04/kWh. PSO also publishes a renewable incentive rate of $0.035/kWh, used in the favorable/upside scenario. For reference, the EIA 2024 Oklahoma industrial average is $0.0584/kWh; Tobe's tariff is below average due to large-power classification and direct industrial service.

For sensitivity, electricity price is modeled from $0.025/kWh (favorable PPA) to $0.055/kWh (approaching EIA state average).

## 5.2 Fixed O&M

| Component | % of CAPEX | Annual Cost (per T-25 at $1,000/kW) | Source |
| --- | --- | --- | --- |
| Routine maintenance | 1.5% | $750 | Industry benchmark |
| Controls/monitoring | 0.5% | $250 | FEED Package scope |
| Insurance & permitting | 1.0% | $500 | Industry benchmark |
| Total fixed O&M | 3.0% | $1,500/year | Benchmarked vs. 2–5% range in deep research report |

3% of CAPEX is at the low end of the 2–5% benchmark range, justified by Tobe's simplified BOP (no cooling system maintenance, no membrane monitoring, no catalyst management).

## 5.3 Water

| Parameter | Value | Source |
| --- | --- | --- |
| City water consumption | 9.0 kg per kg H₂ | ENG_HMB |
| City water cost (Tulsa, OK) | ~$2.50/1,000 gallons | Municipal rate |
| Water cost per kg H₂ | $0.02/kg | — |
| Annual water cost per T-25 (85% CF) | ~$155 | Negligible contribution to LCOH |

## 5.4 Stack Replacement and Degradation

Tobe's membrane-free architecture fundamentally changes the stack replacement calculus:

| Factor | PEM Electrolysis | Tobe Energy |
| --- | --- | --- |
| Primary degradation mode | Membrane thinning, catalyst dissolution | Electrode fouling (cleanable) |
| Replacement interval | 40,000–80,000 operating hours | >80,000 hours (conservative estimate) |
| Replacement cost | 11–35% of installed CAPEX (membrane + catalysts) | <10% of installed CAPEX (stainless steel, no precious metals) |
| Materials replaced | Nafion membrane, Ir/Pt catalysts | Stainless steel electrodes (commodity pricing) |

Modeled assumption (conservative):

Replacement interval: 80,000 operating hours

At 85% capacity factor: ~10.7 years between replacements

Replacement cost: 10% of installed CAPEX ($5,000 per T-25)

Annualized replacement provision: $0.06/kg H₂

Over 20-year economic life: approximately 1 replacement event

This is conservative because:

Stainless steel electrodes do not suffer the chemical degradation that limits PEM membrane life

No rare earth catalyst depletion or poisoning occurs

The system operates at near-ambient temperature (27°C), reducing thermal stress on all components

Electrode fouling can be addressed through cleaning rather than full replacement

### Degradation Model

A simple linear degradation model is applied following the European Hydrogen Observatory LCOH calculator methodology:

| Parameter | Value |
| --- | --- |
| Degradation rate | 0.5% SEC increase per 10,000 operating hours |
| Total degradation over 80,000 hours | ~4% |
| Life-average SEC multiplier | Base SEC × 1.02 (midpoint of degradation curve) |
| Impact on base case LCOH | +$0.04/kg |
| Replacement reset | Stack replacement at 80,000 hours resets degradation to BOL |

At base case SEC of 46.5 kWh/kg, the life-average SEC is 46.5 × 1.02 = 47.43 kWh/kg. The $0.04/kg degradation impact is noted but not incorporated into the headline LCOH to maintain comparability with benchmark TEAs that typically report BOL SEC. The all-in LCOH including degradation is $2.83/kg.

Long-duration degradation testing will commence with the Zeeco ARC deployment in Q4 2026 — Tobe's first commercial installation. Until validated lifetime data is available from sustained commercial operation, the 80,000-hour stack replacement interval and 0.5%/10,000-hour degradation rate are conservative engineering estimates based on the known degradation characteristics of stainless steel electrodes in near-ambient electrochemical environments. These assumptions will be updated as operational data accumulates.

# 6. Revenue and Margin Model

## 6.1 Real Contracted and LOI Pricing

Tobe's pricing is validated by actual contracts and signed letters of intent — not hypothetical market estimates.

Source: TEA_PIPELINE_NARRATIVE.md; Fin_FinancialModel_v4.3.0.xlsx

| Codename | kg/day | $/kg | Status | Annual Revenue |
| --- | --- | --- | --- | --- |
| CARDINAL | 50 | $30 | Contracted | $548K |
| REFINERY | 500 | $15 | LOI signed | $2,738K |
| FORGE | 133 | $25 | LOI signed | $1,214K |
| CAMPUS | 32 | $20 | LOI signed | $234K |
| Subtotal (signed) | 715 | $19.0 avg |  | $4,734K |

Additional pipeline beyond signed LOIs:

| Codename | kg/day | Sector | Status |
| --- | --- | --- | --- |
| HORIZON | 250 | Mobility/Fueling | LOI signed |
| PHOENIX | 1,250 (+5,000 expansion) | E-Fuels | LOI signed |
| TRANSIT | 500 | Public Transit | Full proposal submitted |
| FORGE-STEEL | TBD | Steel Manufacturing | Actively quoting ($7M first site → $49M 7-site) |

Total signed capacity: 2,165 kg/day across 7 sectors

Total pipeline value: $100M+

## 6.2 Gross Margin Analysis

| Selling Price ($/kg) | LCOH Base Case ($/kg) | Gross Margin ($/kg) | Gross Margin (%) |
| --- | --- | --- | --- |
| $15 | $2.79 | $12.21 | 81.4% |
| $20 | $2.79 | $17.21 | 86.1% |
| $25 | $2.79 | $22.21 | 88.8% |
| $30 | $2.79 | $27.21 | 90.7% |

These margins are enabled by the on-site production model: Tobe eliminates the 75–85% of delivered hydrogen cost attributable to transport, storage, and distribution.

## 6.3 The On-Site Advantage

Source: MKRT_CASE_CostAnalysis_OnSiteAdvantage.pdf

The on-site advantage is quantified by an actual market quote:

Airgas cylinder delivery quote (September 2024): Size 300 high-pressure cylinder, 2,500 psi fill, ~0.693 kg H₂ per cylinder, $84.05 per fill = $121.31/kg delivered

Tobe on-site production cost: $2.79/kg (base case LCOH)

Cost reduction vs. cylinder delivery: >97%

Even against lower-cost tube trailer delivery ($15–25/kg for grey hydrogen), Tobe's economics are compelling:

| Delivery Method | Customer Pays | Tobe Offers | Customer Savings |
| --- | --- | --- | --- |
| Cylinder (Airgas) | $121/kg | $20/kg | 83% |
| Tube trailer (grey H₂) | $15–25/kg | $15–25/kg | 0% cost, gains green H₂ |
| On-site PEM (competitor) | $4–7/kg production + CAPEX | $15–25/kg all-inclusive | No CAPEX, no operations burden |

Figure 2: On-Site Production Cost Advantage

The value proposition is that Tobe produces on-site more cheaply than any current delivered hydrogen option while offering green, emission-free hydrogen and eliminating customer CAPEX and operational complexity.

## 6.4 Delivered Cost by End-Use Application

LCOH at electrolyzer outlet is the primary comparability metric. Actual delivered cost to end-use equipment depends on downstream conditioning requirements:

| End-Use Application | Outlet Pressure | Compression Adder | Total Delivered Cost |
| --- | --- | --- | --- |
| Industrial low-pressure (burners, furnaces, process heat) | ~40 psig (outlet) | $0/kg | $2.79/kg |
| Industrial with buffer storage (batch processes) | 200–500 psig | $0.50–1.00/kg | $3.29–3.79/kg |
| Mobility at 350 bar (bus/truck fueling) | 350 bar | $1.50–2.00/kg | $4.29–4.79/kg |
| Mobility at 700 bar (passenger vehicle fueling) | 700 bar | $2.50–3.50/kg | $5.29–6.29/kg |

All currently signed customers are low-pressure industrial users — they consume hydrogen at or near the electrolyzer outlet pressure with no compression required. The January 2026 equipment quote confirms this boundary by listing compression and storage as separate TBD line items.

Compression adder estimates based on DOE compression energy ranges (4.5–7.1 kWh/kg for high-pressure applications) and published station-level TEA data (NREL).

## 6.5 Revenue Streams

The business model includes two complementary revenue streams:

Hydrogen offtake (recurring): Produce and sell H₂ on-site at customer locations

System sales (one-time + service): Sell electrolyzer units directly

System sales backlog:

10× T-25 units: $1.5–2.5M

10× T-125 units: $4.5–6.5M

Total system sales: $6–9M

Source: TEA_PIPELINE_NARRATIVE.md

# 7. Financial Model / DCF

## 7.1 LCOH Calculation Methodology

LCOH is calculated using the Capital Recovery Factor (CRF) method, following the approach recommended by Agora Energiewende and the European Hydrogen Observatory LCOH Calculator:

LCOH = (CAPEX_ann + OPEX_fix + C_replacement + C_electricity + C_water) / H₂_annual

Where:

CAPEX_ann = Installed_CAPEX × CRF

CRF = r(1+r)^n / ((1+r)^n - 1)

H₂_annual = Rated_output × 365 × Capacity_factor

## 7.2 Base Case LCOH Calculation (Single T-25 Unit)

| Parameter | Value | Source |
| --- | --- | --- |
| Rated power | 50 kW | T-25 specification |
| H₂ output | 25 kg/day | ENG_HMB |
| SEC | 46.5 kWh/kg (system-level) | Base case (Section 3.3) |
| Installed CAPEX | $1,000/kW = $50,000 | Section 4.2 |
| Electricity price | $0.04/kWh | PSO large-power industrial tariff |
| Capacity factor | 85% | Validated by signed offtake (2,165 kg/day pipeline) |
| Economic life | 20 years | Conservative; no membrane degradation supports longer life |
| WACC | 8% | Base case per deep research report recommendation |
| Fixed O&M | 3% of CAPEX/year | Section 5.2 |
| Stack replacement | 10% of CAPEX at 80,000 operating hours | Section 5.4 |
| Water cost | $0.02/kg H₂ | Section 5.3 |

Step-by-step calculation:

| Step | Calculation | Result |
| --- | --- | --- |
| Annual H₂ production | 25 × 365 × 0.85 | 7,756 kg/year |
| CRF (8%, 20 years) | 0.08 × 4.661 / 3.661 | 0.1019 |
| Annualized CAPEX | $50,000 × 0.1019 | $5,094/year |
| Fixed O&M | $50,000 × 0.03 | $1,500/year |
| Electricity cost | 7,756 × 46.5 × $0.04 | $14,426/year |
| Water cost | 7,756 × $0.02 | $155/year |
| Replacement provision | $5,000 / 10.7 years (annualized) | $467/year |
| Total annual cost |  | $21,642/year |
| LCOH | $21,642 / 7,756 | $2.79/kg |

### LCOH Cost Breakdown (Base Case)

Figure 3: LCOH Cost Breakdown (Base Case)

Electricity dominates at 67% of LCOH, consistent with the 55–70% benchmark from the deep research report. Including the degradation adjustment (+$0.04/kg, Section 5.4), the all-in LCOH is $2.83/kg.

## 7.3 Three-Scenario LCOH Summary

| Parameter | Downside | Base Case | Upside |
| --- | --- | --- | --- |
| SEC (kWh/kg) | 47.5 | 46.5 | 42.0 |
| Electricity price ($/kWh) | 0.045 | 0.040 | 0.035 (PSO renewable) |
| Installed CAPEX ($/kW) | 1,200 | 1,000 | 800 |
| Capacity factor | 75% | 85% | 90% |
| WACC | 10% | 8% | 6% |
| Fixed O&M (% CAPEX) | 4% | 3% | 2% |
| LCOH ($/kg) | $3.61 | $2.79 | $2.06 |

All three scenarios remain well below the PEM electrolysis benchmark of $4–7/kg.

## 7.4 WACC Sensitivity

| WACC | CRF (20 yr) | Annualized CAPEX | LCOH |
| --- | --- | --- | --- |
| 6% | 0.0872 | $4,360 | $2.70/kg |
| 8% | 0.1019 | $5,094 | $2.79/kg |
| 10% | 0.1175 | $5,873 | $2.89/kg |

LCOH is relatively insensitive to WACC because CAPEX is only ~24% of total cost. A 4-percentage-point swing in WACC changes LCOH by only $0.19/kg.

## 7.5 45V Production Tax Credit Analysis

Tobe qualifies for the Section 45V Clean Hydrogen Production Tax Credit. Final rules under 45VH2-GREET determine the credit tier based on lifecycle emissions. Three scenarios are modeled reflecting compliance uncertainty:

### 45V Credit Scenarios

| Scenario | Credit Value | Qualification Requirements | Compliance Cost | Effective Base LCOH |
| --- | --- | --- | --- | --- |
| Full — Tier 1 | $3.00/kg | Prevailing wage + registered apprenticeship + lifecycle emissions ≤0.45 kg CO₂e/kg H₂ (45VH2-GREET) | $0.10/kg | -$0.11/kg |
| Partial — Tier 2 | $1.00/kg | Lifecycle emissions 0.45–1.5 kg CO₂e/kg H₂; reduced compliance requirements | $0.05/kg | $1.84/kg |
| No PTC | $0/kg | Baseline (credit not secured or not eligible) | $0 | $2.79/kg |

Temporal matching requirements: EAC-based claims require annual matching through 2029 and hourly matching starting 2030. Tobe's grid-connected model in Oklahoma requires an explicit electricity attributes strategy (EACs or direct PPA) for Tier 1 qualification.

Compliance costs ($0.05–0.10/kg) include verification, reporting, EAC procurement, and prevailing wage/apprenticeship administration. These are modeled as a direct LCOH adder rather than absorbed into O&M.

The subsidy value per facility under the Full Tier 1 scenario (based on financial model projections):

$27M per eligible facility over 10-year PTC window

4 eligible facilities identified (Tulsa, OKC, Seattle, Spokane)

Total 45V value: $108M cumulative

Source: Fin_FinancialModel_v4.3.0.xlsx; Tobe_TEA_Narrative_v1.md

Note: 45V PTC qualification requires compliance with prevailing wage and registered apprenticeship requirements under IRA Section 45V(e). The begin-construction deadline is currently 2027 under OBBBA, with a 10-year production credit window. The negative effective LCOH under the Full Tier 1 scenario reflects the PTC's intended policy outcome — incentivizing domestic clean hydrogen production — and should be interpreted as a margin enhancement rather than a sustainable long-term production cost.

## 7.6 Zeeco Phase 1 Deployment Economics (Worked Example)

The Zeeco ARC deployment provides a concrete validation of the LCOH model against a real contracted project with FEED-level engineering.

Source: TOBE FEED Package – Zeeco ARC; TOBE-PFD-001; TEA_PIPELINE_NARRATIVE.md

| Parameter | Value | Source |
| --- | --- | --- |
| Configuration | 12× T-25 units | TOBE FEED Package |
| Total capacity | 300 kg H₂/day | TOBE-PFD-001 |
| Connected load | 576 kW (stacks) + 50 kW (aux) = 626 kW | TOBE FEED Package |
| Annual H₂ at 85% CF | 93,075 kg | 300 × 365 × 0.85 |
| Annual electricity | 4,327,988 kWh | 93,075 × 46.5 |
| Electricity cost | $173,120/year | At $0.04/kWh |
| Installed CAPEX (est.) | $626,000 | 626 kW × $1,000/kW |
| Annualized CAPEX | $63,789 | CRF = 0.1019 |
| Fixed O&M | $18,780/year | 3% of CAPEX |
| Water | $1,862/year | 93,075 × $0.02 |
| Replacement provision | $5,830/year | 10% of CAPEX annualized |
| Total annual cost | $263,381 |  |
| LCOH | $2.83/kg | Slightly above single-unit due to auxiliary load |
| Revenue at $30/kg | $2,792,250/year | Contracted Zeeco price |
| Gross profit | $2,528,869/year |  |
| Gross margin | 90.6% |  |
| CAPEX payback | ~3 months | $626K / $2.53M annual GP |

This worked example validates that the unit-level LCOH model scales predictably to multi-unit deployments, and that the contracted Zeeco price of $30/kg produces exceptional margins even at the facility level.

## 7.7 DCF / Revenue Projections

The financial model projects seven-year revenue growth driven by facility deployment, equipment sales scale-up, and the expanding customer pipeline:

| Metric | Year 1 (2026) | Year 2 (2027) | Year 3 (2028) | Year 4 (2029) | Year 5 (2030) | Year 6 (2031) | Year 7 (2032) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H₂ production revenue | $12.4M | $24.1M | $46.1M | $86.6M | $139.6M | $267.2M | $270.0M |
| Equipment sales revenue | $6.5M | $26.9M | $82.5M | $234.1M | $470.6M | $641.3M | $787.5M |
| Equipment services revenue | — | $0.4M | $2.3M | $8.5M | $26.6M | $66.7M | $130.1M |
| 45V PTC revenue | $1.5M | $2.9M | $5.3M | $5.4M | $5.4M | $5.4M | $5.4M |
| Total revenue | $20.4M | $54.3M | $136.2M | $334.6M | $642.2M | $980.6M | $1,193.0M |
| Gross margin | 77.7% | 70.4% | 68.6% | 68.0% | 68.7% | 71.0% | 69.6% |
| EBITDA | $12.9M | $32.2M | $80.9M | $201.0M | $394.2M | $631.2M | $755.7M |
| EBITDA margin | 63.3% | 59.3% | 59.4% | 60.1% | 61.4% | 64.4% | 63.3% |
| Net income | $8.7M | $21.7M | $56.3M | $141.5M | $280.6M | $447.1M | $535.1M |
| Headcount (FTEs) | 18 | 28 | 37 | 53 | 102 | 120 | 138 |
| Operating expense | $2.9M | $6.0M | $12.6M | $26.4M | $46.8M | $65.1M | $75.1M |
| Year-end cash | $12.7M | $77.1M | $149.9M | $330.4M | $603.6M | $1,240.0M | $1,738.8M |

Revenue mix shifts from H₂-production-dominant (61% in Year 1) to equipment-sales-dominant (66% by Year 7), reflecting the transition from operated hydrogen-as-a-service deployments to a scaled equipment manufacturing business. Equipment services revenue (recurring maintenance contracts at 75% attach rate) emerges as a growing third stream from Year 2 onward.

Fundraising trajectory:

| Round | Timing | Amount | Purpose |
| --- | --- | --- | --- |
| Seed | May 2026 | $10M ($7.5M equity + $2.5M debt) | First deployments, FEED execution |
| Series A | June 2027 | $40–46M | Multi-site buildout |
| Series B | March 2029 | $60–85M | National scale-up |
| Series C | March 2031 | $150–225M | Full commercialization |

Exit valuation framework: $2.6–3.3B at 8–10× revenue multiple

# 8. Sensitivity Analysis

## 8.1 Tornado Chart — Single-Variable LCOH Sensitivity

Each variable is varied independently while holding all others at base case ($2.79/kg at $0.04/kWh).

Figure 4: Single-Variable LCOH Sensitivity (Tornado Chart)

Key finding: Electricity price is the dominant sensitivity driver, consistent with the report's finding across all nine exemplar TEAs. A swing from $0.025 to $0.055/kWh changes LCOH by $1.40/kg. Even at the most expensive electricity modeled ($0.055/kWh), Tobe's LCOH ($3.49/kg) remains below the PEM benchmark floor of $4/kg.

## 8.2 Multi-Variable Scenario Analysis

| Scenario | Elec ($/kWh) | CF | CAPEX ($/kW) | SEC (kWh/kg) | WACC | LCOH ($/kg) | vs. PEM Benchmark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Best case | 0.025 | 95% | 700 | 42.0 | 6% | $1.54 | 61–78% below |
| Favorable | 0.035 | 90% | 800 | 42.0 | 6% | $2.06 | 48–71% below |
| Base case | 0.040 | 85% | 1,000 | 46.5 | 8% | $2.79 | 30–60% below |
| Conservative | 0.045 | 75% | 1,200 | 47.5 | 10% | $3.61 | 10–48% below |
| Worst case | 0.055 | 50% | 1,500 | 50.0 | 10% | $5.61 | At PEM parity |

Tobe's LCOH exceeds PEM benchmarks only in an extreme worst-case scenario combining the highest electricity price, lowest utilization, highest CAPEX, worst efficiency, and highest cost of capital — a scenario that is unlikely given the signed offtake pipeline and Oklahoma's industrial electricity rates.

## 8.3 Capacity Factor Sensitivity

The signed pipeline of 2,165 kg/day validates high utilization targets. However, LCOH is moderately sensitive to capacity factor because fixed costs (CAPEX annuity, O&M) are spread over fewer production kilograms at lower utilization:

| Capacity Factor | Annual H₂ (kg) | LCOH ($/kg) | Gross Margin at $25/kg |
| --- | --- | --- | --- |
| 30% | 2,738 | $4.35 | 82.6% |
| 50% | 4,563 | $3.39 | 86.4% |
| 75% | 6,844 | $2.90 | 88.4% |
| 85% | 7,756 | $2.79 | 88.8% |
| 95% | 8,669 | $2.70 | 89.2% |

Even at 30% capacity factor — a level typical of intermittent renewable-only systems — Tobe's LCOH ($4.35/kg) remains within the PEM benchmark range. Gross margins exceed 82% at all utilization levels when selling at $25/kg.

## 8.4 Hydrogen Selling Price Sensitivity

| Selling Price ($/kg) | Gross Margin (Base LCOH) | Annual Gross Profit per T-25 (85% CF) | Payback Period |
| --- | --- | --- | --- |
| $15 | 81.4% | $94,661 | <7 months |
| $20 | 86.1% | $133,441 | <5 months |
| $25 | 88.8% | $172,221 | <4 months |
| $30 | 90.7% | $211,001 | <3 months |

At all contracted/LOI price points, the T-25 unit pays back its installed capital ($50,000 at base case) in under 12 months.

# 9. Competitive Comparison

## 9.1 Production Cost Comparison

| Technology | SEC (kWh/kg) | Installed CAPEX ($/kW) | LCOH at Outlet ($/kg) | Source |
| --- | --- | --- | --- | --- |
| PEM electrolysis | 50–58 | 1,400–2,500 | $4.40–7.90 | DOE 2024 program record |
| Alkaline electrolysis | 51–56 | 500–1,000 | $4.00–6.00 | IRENA 2020; IEA 2023 |
| Grey hydrogen (SMR) | N/A (NG feedstock) | — | $1.00–2.00 | DOE reference |
| Tobe Energy (base) | 46.5 | 1,000 | $2.79 | This analysis |
| Tobe Energy (upside) | 42.0 | 800 | $2.06 | This analysis |

## 9.2 Wall-to-Plug Efficiency Comparison

This comparison requires careful framing. Most published PEM efficiencies are stack efficiencies (DC → H₂), not wall-to-plug (AC → H₂). Tobe reports wall-to-plug exclusively. See Section 3.2.1 for the stack vs. system SEC boundary clarification.

| Metric | Tobe Energy (System) | Tobe Energy (Stack) | PEM (Wall-to-Plug) | PEM (Stack Only) |
| --- | --- | --- | --- | --- |
| SEC (kWh/kg) | 46–48 | 42.2 | 50–58 | 42–51 (DC basis) |
| LHV efficiency | 70–72% | 79% | 57–67% | 65–80% |
| HHV efficiency | 83–85% | 93.4% | 67–79% | 77–95% |

Source: ENG_EfficiencyCalculations.xlsx (Tobe); TOBE-PFD-001 (stack SEC); DOE 2024, NREL 2024, IRENA 2020 (PEM)

Tobe's system-level wall-to-plug efficiency of 70–72% LHV exceeds the typical PEM wall-to-plug range of 57–67% LHV. Tobe's stack-level SEC of 42.2 kWh/kg is comparable to the best PEM stack-only numbers — but Tobe achieves this with stainless steel at a fraction of the cost.

## 9.3 Delivered Cost Comparison (The Comparison That Matters)

For end customers, the relevant comparison is total cost of hydrogen at point of use, not production cost:

| Source | Production Cost | Delivery Cost | Total Delivered |
| --- | --- | --- | --- |
| Grey H₂ (SMR) + tube trailer | $1–2/kg | $13–23/kg | $15–25/kg |
| Grey H₂ (SMR) + cylinder (Airgas) | $1–2/kg | $119–120/kg | $121/kg |
| PEM (on-site, self-owned) | $4–7/kg | $0 | $4–7/kg + CAPEX risk |
| PEM (centralized) + delivery | $4–7/kg | $5–10/kg | $9–17/kg |
| Tobe (on-site, as-a-service) | $2.79/kg | $0 | $15–30/kg all-in |

Tobe's value proposition: customers pay $15–30/kg for on-site green hydrogen, which is at or below their current delivered grey hydrogen cost, with zero CAPEX commitment, zero operational burden, and the environmental benefit of emission-free production.

## 9.4 Structural Advantages Summary

| Advantage | Impact | Durability |
| --- | --- | --- |
| No membrane | Lower CAPEX, no degradation-driven replacement | Permanent — fundamental architecture |
| No rare earth catalysts | Lower CAPEX, no supply chain risk (Ir/Pt) | Permanent — stainless steel construction |
| Near-ambient operation (27°C) | No cooling BOP, lower energy consumption | Permanent — thermodynamic characteristic |
| TDS tolerance (25–10,000+ ppm) | Standard RO vs. UPW/DI, broader site applicability | Permanent — demonstrated across conditions |
| On-site production model | Eliminates 75–85% of delivered H₂ cost for low-pressure uses | Structural — business model advantage |
| Wall-to-plug efficiency leadership | Lower electricity cost per kg at the dominant cost driver | Improvable — optimized topology shows 94% HHV path |

# 10. Conclusions

## 10.1 Tobe's measured efficiency already beats PEM benchmarks

Across four test configurations spanning two power levels and two water quality conditions, Tobe's electrolyzer delivers 70–72% LHV wall-to-plug efficiency (system SEC of 46–48 kWh/kg; stack SEC of 42.2 kWh/kg). This exceeds the typical PEM wall-to-plug range of 57–67% LHV, using conservative bench power supply conditions. The demonstrated upside (94% HHV with resonant LLC topology) suggests significant headroom for further improvement.

Source: ENG_EfficiencyCalculations.xlsx; ENG_PreCommit_SampleOperatingData.xlsx; TOBE-PFD-001

## 10.2 On-site production eliminates 75–85% of delivered hydrogen cost

DOE data and the Airgas market quote ($121/kg delivered) confirm that transport, storage, and distribution dominate hydrogen cost for end users. Tobe's on-site model eliminates these costs entirely for low-pressure industrial applications, allowing production-cost LCOH of $2.79/kg to translate directly into customer value at $15–30/kg selling prices.

Source: MKRT_CASE_CostAnalysis_OnSiteAdvantage.pdf; TEA_PIPELINE_NARRATIVE.md

## 10.3 Real contracted pricing validates >81% gross margins

Gross margins of 81–91% are calculated from real contracted prices ($30/kg Zeeco) and signed LOIs ($15–25/kg across four customers) against a measured production cost basis. The weighted average selling price across signed commitments is ~$19/kg.

Source: TEA_PIPELINE_NARRATIVE.md; Fin_FinancialModel_v4.3.0.xlsx

## 10.4 Technology advantages drive structural CAPEX advantage

Equipment cost of $500/kW (January 2026 quote) is 37–63% below PEM equipment-only cost of $800–1,350/kW (NREL 2024). Fully installed at $1,000/kW, Tobe remains below the PEM uninstalled cost. This advantage stems from elimination of the membrane, rare earth catalysts, and active cooling system, combined with commodity stainless steel construction.

Source: January 2026 quote (Jan 2026); TOBE-FEED-PKG-001; NREL 2024 (deep research report)

## 10.5 $100M+ pipeline across 7 sectors proves horizontal market applicability

With 2,165 kg/day of signed capacity across industrial, manufacturing, academic, mobility, e-fuels, transit, and EPC, Tobe has demonstrated that the on-site hydrogen production model addresses a broad, horizontal market — not a single niche application.

Source: TEA_PIPELINE_NARRATIVE.md

## 10.6 Investment Thesis

Tobe Energy offers investors:

Production economics that beat all current electrolysis benchmarks ($2.79/kg vs. $4–7/kg PEM)

Gross margins of 81–91% validated by real commercial contracts

A $100M+ pipeline across seven industry sectors with geographic diversity

Technology differentiation that is structural (no membrane, no rare earths, near-ambient operation) rather than incremental

Policy tailwinds via the 45V PTC (up to $108M cumulative value across four facilities under Full Tier 1)

Capital efficiency with sub-12-month payback on deployed units

# Appendix A: Source File Reference

| File | Description | Key Data Used |
| --- | --- | --- |
| ENG_PreCommit_SampleOperatingData.xlsx | 22,103 data points, 5-sec intervals, 6kW prototype | System SEC (46.08 kWh/kg), stack temp (27–28°C), operating pressure |
| ENG_EfficiencyCalculations.xlsx | Four test runs across 2 power levels, 2 TDS conditions | LHV efficiency (69–72%), HHV efficiency (82–85%) |
| ENG_HMB_HeatAndMaterialBalance.xlsx | T-25 unit H&MB + multi-unit scaling | Water consumption, energy inputs, material flows |
| ENG_GRANT_NavySBIR_N242-070.pdf | Published efficiency data (federal grant) | Efficiency validation, TDS tolerance data |
| TOBE FEED Package – Zeeco ARC.pdf | Real EPC scope and installed cost data | System boundary, equipment scope, battery limits |
| TOBE-FEED-PKG-001.pdf | Detailed FEED engineering package | Materials (304 SS), codes/standards, electrical specs |
| TOBE-PFD-001.pdf | Process flow diagram | Stack SEC 42.2 kWh/kg deployment basis |
| Quote (Jan 2026) | Equipment pricing at breakeven | T-25 $25K ($500/kW), T-125 $120K ($480/kW), scope definition |
| MKRT_CASE_CostAnalysis_OnSiteAdvantage.pdf | On-site vs. delivered cost analysis | Airgas quote ($121/kg), market segmentation |
| Fin_FinancialModel_v4.3.0.xlsx | Latest financial model (v4.3.0) | Revenue projections, unit pricing, 45V analysis |
| FINAL_Tobe_Seed_Financial_Model_v853.xlsx | Seed financial model (v8.5.3) | Facility CAPEX, headcount, OpEx trajectory |
| TEA_PIPELINE_NARRATIVE.md | Customer pipeline, pricing, LOIs | Contracted pricing, utilization validation |
| TEA_Deep_Research_Report.md | 9 TEA exemplars, methodology guide | LCOH methodology, benchmark parameters, sensitivity approach |
| Tobe_TEA_Narrative_v1.md | Prior TEA narrative | Unit economics, fundraising trajectory |
| NOTES_operating_data.md | Context for operating data | Critical test parameters |
| NOTES_efficiency_calculations.md | Context for efficiency data | SBIR label correction, scenario definitions |
| NOTES_heat_material_balance.md | Context for H&MB data | Zeeco 12-unit revision, deployment timeline |

# Appendix B: Glossary

| Term | Definition |
| --- | --- |
| BOP | Balance of Plant — auxiliary systems (power electronics, water treatment, thermal management, controls) |
| CF | Capacity Factor — fraction of rated output achieved annually |
| CRF | Capital Recovery Factor — annualization factor for CAPEX |
| FLH | Full Load Hours — equivalent hours at rated capacity per year |
| HHV | Higher Heating Value of hydrogen (39.41 kWh/kg = 142 MJ/kg) |
| LCOH | Levelized Cost of Hydrogen — annualized total cost per kg produced |
| LHV | Lower Heating Value of hydrogen (33.33 kWh/kg = 120 MJ/kg) |
| LLC | Inductor-Inductor-Capacitor — resonant power converter topology |
| PEM | Proton Exchange Membrane — conventional electrolysis technology |
| PTC | Production Tax Credit — Section 45V of the Inflation Reduction Act |
| RO | Reverse Osmosis — water purification technology |
| SEC | Specific Energy Consumption — kWh per kg H₂ produced |
| SMR | Steam Methane Reforming — conventional hydrogen production from natural gas |
| TDS | Total Dissolved Solids — measure of water impurity (ppm) |
| WACC | Weighted Average Cost of Capital — discount rate for DCF analysis |